Annex 1

Data elements and format

Header refers to items that should have a single value established at trigger or before the event.  Data in pre-trigger shall be reported at least 2 values per second for at least 5 seconds before a trigger.

Table 1 - List of data elements

| Data element | Condition for requirement | Recording interval/time (relative to time zero trigger event) | Data sample rate (samples per second) | Minimum range | Accuracy | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Event Data Recording Complete | Mandatory | [Following other data] | N/A | Yes or No | N/A | Yes or No | All 3.3.1 triggers Planar VRU Rollover |
| Event Data Recording Complete | Mandatory | [Following other data] | [N/A] | [Yes or No] | [N/A] | [Yes or No] | All 3.3.1 triggers Planar VRU Rollover |
| [Engine hours event] | Mandatory | [-1.0 sec] | [N/A] | [0 to 210,554,060.75 hr] | [±0.05 hr] | [0.05 hr] | All 3.3.1 triggers |
| [Engine hours download] | Mandatory | [At time of download] | [N/A] | [0 to 210,554,060.75 hr] | [±0.05 hr] | [0.05 hr] | All 3.3.1 triggers |
| [Odometer] |  |  |  |  |  |  | All 3.3.1 triggers |
| Odometer |  |  |  |  |  |  | All 3.3.1 triggers |
| HD EDR Unit Hardware Part # | Mandatory | N/A | N/A | N/A | N/A | N/A | N/A |
| HD EDR Unit Software Part # | Mandatory | N/A | N/A | N/A | N/A | N/A | N/A |
| Vehicle Make | Mandatory | [Event] | [N/A] | [N/A] | [N/A] | [Alphanumeric type-variant-version code] | All 3.3.1 triggers |
| Vehicle Make | Mandatory |  |  |  |  |  |  |
| Vehicle Model | Mandatory |  |  |  |  |  | All 3.3.1 triggers |
| Vehicle Model | Mandatory |  |  |  |  |  | All 3.3.1 triggers |
| [Conversion factor between drive shaft rpm and vehicle speed] |  |  |  |  |  |  | All 3.3.1 triggers |
| [Conversion factor between drive shaft rpm and vehicle speed] |  |  |  |  |  |  | All 3.3.1 triggers |
| Trigger Thresholds | Mandatory | [Event] | [N/A] | [8.0 to 22.5 km/h/s] | [N/A] | [0.1 km/h/s] | All 3.3.1 triggers |
| Trigger Threshold Activated | Mandatory | [Event] | [N/A] | [N/A] | [N/A] | [Deceleration, Last stop, AEB, ABS, Safety restraint system, VRU secondary safety system, Other] | All 3.3.1 triggers |
| Vehicle configuration [pending more info from SAE expert] |  |  |  |  |  |  | All 3.3.1 triggers |
| Vehicle configuration [pending more info from SAE expert] |  |  |  |  |  |  | All 3.3.1 triggers |
| [Vehicle Safety System Manifest] | [Mandatory] | [Event] | [N/A] | [N/A] | [N/A] | [List of fitted active safety and accident avoidance systems] | All 3.3.1 triggers |
| [Vehicle Safety System Manifest] |  |  |  |  |  |  | All 3.3.1 triggers |
| [Ignition cycle crash] | [Mandatory] | [-1.0 sec] | [N/A] | [0 to 60,000] | [±1 cycle] | [1 cycle] | All 3.3.1 triggers |
| [Ignition cycle download] | [Mandatory] | [At time of download] | [N/A] | [0 to 60,000] | [±1 cycle] | [1 cycle] | All 3.3.1 triggers |
| Vehicle Speed | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [0 to 250 km/h] | [±1 km/h] | [1 km/h] | All 3.3.1 triggers |
| Vehicle Speed | Mandatory | [-5.0 to 0 sec] | [2] | [0 to 250 km/h] | [±1 km/h] | [1 km/h] | All 3.3.1 triggers Planar VRU |
| Retarder Torque Mode | Mandatory |  |  |  |  |  | All 3.3.1 triggers |
| Retarder Torque Mode | Mandatory | -5.0 to 0 sec | 2 | NA | NA | NA | All 3.3.1 triggers |
| Brake Status – Parking | Mandatory | [-5.0 to 0 sec] | [2] | [On or Off] | [NA] | [On or Off] | All 3.3.1 triggers |
| Brake Status – Parking | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [On or Off N/A] | [NA] | [On or Off] | All 3.3.1 triggers |
| Brake Status – Service | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [On or Off N/A] | [NA] | [On or Off] | All 3.3.1 triggers Planar VRU Rollover |
| Brake Status – Service | Mandatory | [-5.0 to 0 sec] | [2] | [On or Off] | [NA] | [On or Off] | Planar VRU Rollover All 3.3.1 triggers |
| Engine Speed | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [0-10,000 rpm] | [±100 rpm] | [100 rpm][ | All 3.3.1 triggers Planar Rollover |
| Engine Speed | Mandatory | [-5.0 to 0 sec] | [2] | [0-10,000 8,031 rpm] | [±100 rpm] | [100 rpm][ | All 3.3.1 triggers Planar Rollover |
| Engine Load | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [0 to 125%] | [±5%] | [1%] | All 3.3.1 triggers |
| Engine Load | Mandatory | -5.0 to 0 sec | 2 | -125 – 125 % | ±5% | 1% | All 3.3.1 triggers |
| Accelerator Pedal Position | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [0 to 100 %] | [±5%] | [1%] | All 3.3.1 triggers Planar Rollover VRU |
| Accelerator Pedal Position | Mandatory | [-5.0 to 0 sec] | [2] | [0 to 100 %] | [±5%] | [1%] | All 3.3.1 triggers Planar Rollover VRU |
| ABS Brake Control Status – Tractor/Truck Motor vehicle | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [Faulted, Non-Engaged, Engaged N/A] | [NA] | [Faulted, Non-Engaged, Engaged Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers Planar VRU Rollover |
| ABS Brake Control Status – Tractor/Truck | Mandatory | [-5.0 to 0 sec] | [2] | [Faulted, Non-Engaged, Engaged] | [NA] | [Faulted, Non-Engaged, Engaged] | All 3.3.1 triggers Planar VRU Rollover |
| ABS Brake Control Status – Trailer | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [Faulted, Non-Engaged, Engaged N/A] | [NA] | [Faulted, Non-Engaged, Engaged Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers Planar VRU Rollover |
| ABS Brake Control Status – Trailer | Mandatory | [-5.0 to 0 sec] | [2] | [Faulted, Non-Engaged, Engaged] | [NA] | [Faulted, Non-Engaged, Engaged] | All 3.3.1 triggers Planar VRU Rollover |
| Adaptive Cruise Control Mode | Mandatory | [-5.0 to 0 second relative to time zero] | [2] | [NA Off (Standby, enabled, ready for activation) Speed control active Distance control active] | [NA] | [Engaged, Faulted, Off, Non-Engaged Off Speed control active Distance control active] | All 3.3.1 triggers Planar VRU Rollover |
| ACC Mode | Mandatory | [-5.0 to 0 second relative to time zero -20 to +10 sec] | [2 10] | [NA] | [NA] | [Engaged, Faulted, Off, Non-Engaged Off, Faulted, On not controlling, On controlling] | All 3.3.1 triggers Planar VRU Rollover |
| ACC Set Distance |  |  |  |  |  |  | All 3.3.1 triggers |
| Cruise Control States | Mandatory | [-5.0 to 0 second -20 to +10 sec] | [2 10] | [NA] | [NA] | [Engaged, Faulted, Off, Non-Engaged Off, Faulted, On not controlling, On controlling] | All 3.3.1 triggers Planar VRU Rollover |
| Cruise Control States | Mandatory | [-5.0 to 0 second ] | [2] | [NA Off/Disabled Hold Accelerate Decelerate Resume Set Accelerator Override Faulted] | [NA] | [Engaged, Faulted, Off, Non-Engaged Same as range] | All 3.3.1 triggers Planar VRU Rollover |
| Automatic Emergency Braking | Mandatory | [-5.0 to 0 second relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Deactivated, On but Non-Engaged, Engaged Off, Faulted, On not warning/intervening, On warning, On intervening] | All 3.3.1 triggers Planar VRU Rollover |
| Automatic Emergency Braking | Mandatory | [-5.0 to 0 second relative to time zero] | [2] | [N/A system is not ready system is temporarily not available system is deactivated by driver system is ready and activated driver overrides system forward collision warning active forward collision warning with braking forward collision emergency braking active] | [N/A] | [Faulted, Deactivated, On but Non-Engaged, Engaged Same as range] | All 3.3.1 triggers Planar VRU Rollover |
| Time to Collision with Relevant Object [not required] |  |  |  |  |  |  |  |
| Speed of Forward vehicle [not required] |  |  |  |  |  |  |  |
| Distance to forward vehicle [not required] |  |  |  |  |  |  |  |
| Lane Departure Warning System State Status | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not warning, On – Warning left, On – Warning right] | All 3.3.1 triggers Planar Rollover |
| Lane Departure Warning System State | Mandatory | [-5.0 to 0 sec] | [2] | [N/A Active - Inactive] | [N/A] | [Faulted, Off, On but not warning, On – Warning left, On – Warning right Active - Inactive] | All 3.3.1 triggers Planar Rollover |
| Steering wheel angle | Mandatory | [-5.0 to 0 sec -20 to +10 sec] | [2 10] | [-250 deg CW to + 250 deg CCW] | [±5%] | [±1%] | All 3.3.1 triggers Planar VRU Rollover |
| Steering wheel angle | Mandatory | [-5.0 to 0 sec] | [2] | [-250 deg CW to + 250 deg CCW -31 to 31 rad] | [±5% ±0.4 rad] | [±1% 0.2 rad] | All 3.3.1 triggers [Planar VRU Rollover |
| Rollover Protection Engine Control | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers |
| Stability Control System Braking [Rollover protection Engine Control] | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers |
| Stability Control System Braking | Mandatory | -5.0 to 0 sec | 2 | Active – Inactive (foundation braking) | NA | Active - Inactive | All 3.3.1 triggers |
| Yaw Control Engine Retarder Control | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers |
| Yaw Control Brakinge Control | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not intervening, On intervening] | All 3.3.1 triggers |
| Vehicle Dynamic Control System State | Mandatory |  |  |  |  |  | All 3.3.1 triggers |
| Vehicle Dynamic Control System State Stability Control System State | Mandatory | -5.0 to 0 sec | 2 | Active - Inactive | NA | Active - Inactive | All 3.3.1 triggers |
| Blind Spot Warning System Status (Vehicle detection) | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Faulted, Off, On not warning, On warning left, On warning right] | All 3.3.1 triggers |
| Blind Spot Warning System Status | Mandatory | -5.0 to 0 sec | 2 | Active - Inactive | NA | Active - Inactive | All 3.3.1 triggers |
| Crash Type [Impact Mitigation] System Activation Notification | Mandatory | Event | N/A | N/A | N/A | Frontal crash, Rear crash, Side crash (left), Side crash (right), Rollover | All 3.3.1 triggers |
| Safety Seat Belt Status (Driver) | Mandatory | [-1.0 sec] | [N/A] | [Fastened, not Fastened N/A] | [N/A] | [Fastened, not Fastened] | [All 3.3.1 triggers] |
| Seat Belt Status (Driver) | Mandatory | [-1.0 sec] | [N/A] | [Fastened, not Fastened] | [N/A] | [Fastened, not Fastened] | [All 3.3.1 triggers] |
| Safety Seat Belt Status (Passenger) | Mandatory | [-1.0 sec] | [N/A] | [Fastened, not Fastened N/A] | [N/A] | [Fastened, not Fastened] | [All 3.3.1 triggers] |
| Seat Belt Status (Front Outboard Passenger) | Mandatory | [-1.0 sec] | [N/A] | [Fastened, not Fastened] | [N/A] | [Fastened, not Fastened] | [All 3.3.1 triggers] |
| Safety Restraint System Status | Mandatory | [Event] | [N/A] | [N/A] | [N/A] | [Record for each pre-tensioner and airbag separately: Off, Faulted, Deployed] | All 3.3.1 triggers |
| Safety Restraint System Status | Mandatory | -5.0 to 0 sec | 2 | Active - Inactive | NA | Active - Inactive | All 3.3.1 triggers |
| VRU Alert Secondary Safety System Status | Mandatory | [-1.1 to 0 relative to time zero Event] | [N/A] | [N/A] | [N/A] | [On or Off Faulted, Deployed] | All 3.3.1 triggers VRU |
| VRU Alert | Mandatory | [-1.1 to 0 relative to time zero -5.0 to 0 sec] | [N/A 2] | [N/A Active, Inactive] | [N/A] | [On or Off Active, Inactive] | All 3.3.1 triggers VRU |
| VRU Alert - Left Side | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not informing/warning/intervening, On informing, On warning, On intervening] | All 3.3.1 triggers |
| VRU Alert - Right Side | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not informing/warning/intervening, On informing, On warning, On intervening] | All 3.3.1 triggers |
| VRU Alert - Front | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not informing/warning/intervening, On informing, On warning, On intervening] | All 3.3.1 triggers |
| VRU Alert - Rear | Mandatory | [-20 to +10 sec] | [10] | [N/A] | [N/A] | [Off, Faulted, On not informing/warning/intervening, On informing, On warning, On intervening] | All 3.3.1 triggers |
| [Delta-V, longitudinal] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz with sufficient range and resolution to calculate delta-v with required accuracy] | [0 to 250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [100] | [-100 km/h to + 100 km/h.] | [±10%] | [1 km/h.] | [Planar] |
| [Maximum delta-V, longitudinal] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [-100 km/h to + 100 km/h.] | [±10%] | [1 km/h.] | Occupant restraint system (planar events) |
| [Maximum delta-V, longitudinal] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [-100 km/h to + 100 km/h.] | [±10%] | [1 km/h.] | Occupant restraint system (planar events) Planar |
| [Time, maximum delta-V, longitudinal] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter.] | [±3 ms] | [2.5 ms.] | Occupant restraint system (planar events) |
| [Time, maximum delta-V, longitudinal] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter.] | [±3 ms] | [2.5 ms.] | Occupant restraint system (planar events) Planar |
| [Delta-V lateral] | [Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz with sufficient range and resolution to calculate delta-v with required accuracy] | [0 to 250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [100] | [-100 km/h to + 100 km/h.] | [±10%] | [1 km/h.] | [Planar] |
| [Maximum delta-V, lateral] | [Mandatory - not required if lateral acceleration recorded at ≥500 Hz ] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [-100 km/h to + 100 km/h.] | [±10%] | [1 km/h.] | Occupant restraint system (Planer events) |
| [Time, maximum delta-V, lateral] | [Mandatory - not required if lateral acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter.] | [±3 ms] | [2.5 ms.] | Occupant restraint system (planar events) |
| [Time for maximum delta-V, resultant] | [Mandatory - not required if lateral acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter.] | [±3 ms] | [2.5 ms.] | Occupant restraint system (planar events |
| [Time for maximum delta-V, resultant] | [Mandatory - not required if relevant acceleration recorded at ≥500 Hz] | [0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter.] | [N/A] | [0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter.] | [±3 ms] | [2.5 ms.] | Occupant restraint system (planar events) |
| [Roll angle] | [If recorded] | [0 to at least 250ms -20 to +10 sec] | [10] | [-1080 deg to + 1080 deg.] | [±10%] | [10 deg.] | Occupant restraint system (rollover events) |
| [Roll rate] | [Mandatory if fitted and used for rollover protection system control algorithm] | [0 to at least 250ms -20 to +10 sec] | [10] | [-240 to +240 deg/sec] | [±10%] | [4 deg/sec] | Occupant restraint system (rollover events) |
| [Longitudinal Acceleration (pre-crash low frequency)] | [Mandatory] | [-5.0 to 0 second relative to time zero -20 to +10 sec] | [2 10] | [-1.5g to +1.5g] | [±10%] | [0.1g] | All 3.3.1 triggers |
| [Lateral Acceleration (pre-crash low frequency)] | [Mandatory] | [-5.0 to 0 second relative to time zero -20 to +10 sec] | [2 10] | [-1.5g to +1.5g] | [±10%] | [0.1g] | All 3.3.1 triggers |
| [Longitudinal Acceleration (pre-crash] | [Mandatory] | [-5.0 to 0 second relative to time zero] | [2] | [-1.5g to +1.5g] | [±10%] | [0.1g] | All 3.3.1 triggers Planar VRU |
| [Lateral Acceleration (pre-crash | [Mandatory] | [-5.0 to 0 second relative to time zero] | [2] | [-1.5g to +1.5g] | [±10%] | [0.1g] | All 3.3.1 triggers Planar VRU |
| Accident emergency call system status | Mandatory | [Event] | [N/A] | [N/A] | [N/A] | [Faulted, On but emergency call not automatically triggered, On – Emergency call automatically triggered] | Occupant restraint system |
| Tyre pressure monitoring system warning lamp status | Mandatory | [-1.0 second relative to time zero] | [N/A] | [N/A] | [N/A] | [On, Off] | All 3.3.1 triggers |
| [Safety belt status, rear passengers] | [Mandatory] | [-1.0 sec] | [N/A] | [Fastened, not Fastened N/A] | [N/A] | [Fastened, not Fastened [Number of fastened rear passenger safety belts and number of occupied rear passenger seats] | All 3.3.1 triggers |
| [Safety belt status mid-position front] | [Mandatory] | [-1.0 sec] | [N/A] | [Fastened, not Fastened N/A] | [N/A] | [Fastened, not Fastened] | All 3.3.1 triggers Planar Rollover |
| [Moving Off Information System Status (M2, 3 and N2, 3)] |  |  |  |  |  |  | All 3.3.1 triggers |
| [VRU Proximity Sensing system reversing motion status (M2, 3, and N2,3)] |  |  |  |  |  |  | All 3.3.1 triggers |
| [Yaw Rate] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [-75 to +75 degrees/second] | [±10% of the full range of the sensor] | [0.1 ???] | Occupant restraint system (planar events) Planar Rollover |
| [Corrective steering function status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | N/A | N/A | [Faulted, Off, On but not engaged, engaged [intervening, On intervening] | All 3.3.1 triggers Planar Rollover |
| [Emergency steering function status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |
| [Automatically commanded steering function category A status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |
| [Automatically commanded steering function category B status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |
| [Automatically commanded steering function category C status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |
| [Automatically commanded steering function category D status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |
| [Automatically commanded steering function category E status] | [Mandatory] | [-5 to 0 seconds relative to time zero -20 to +10 sec] | [2 10] | [N/A] | [N/A] | [Faulted, Off, On but not engaged, engaged [controlling, On controlling] | All 3.3.1 triggers Planar Rollover |

| Data Element | Description |
| --- | --- |
| [Event Data Recording Complete] | [This data indicates whether a complete set of data that the event data recording device is designed to capture was successfully recorded by and stored in the device.] |
| Event Date [exclude from scope] | The date when the event occurred |
| [Engine Hours] | The number of hours that the engine has been operating from the time of control unit first use to the time of the event trigger |
| [Odometer] | Total vehicle distance at the time of the event trigger |
| Latitude [exclude from scope] | Vehicle position per GNSS at the time of the event |
| Longitude [exclude from scope] | Vehicle position per GNSS at the time of the event |
| Event Time [exclude from scope] | The time when the event occurred |
| HD EDR Unit Hardware Part # | The part number for the HD EDR unit |
| HD EDR Unit Software Part # | The part number/software version number for the HD EDR software |
| Vehicle Make | The name of the vehicle manufacturer |
| Vehicle Model | The vehicle’s model name/number |
| Rear Axle Ratio [pending more info from SAE expert] | Ratio of transmission output shaft speed to tire rotation rate |
| Tire Size [pending more info from SAE expert] | Tire size in revolutions per km |
| Trigger Thresholds | Lists the currently configured trigger threshold(s). |
| Trigger Threshold Activated | Indicates which Trigger Threshold was activated to cause the recording the event. |
| Vehicle Configuration[pending more info from SAE expert] | PGN 65259 and PGN 65242 for available ECU HW and SW part number (needs further clarification from SAE expert) |
| [Vehicle Safety System Manifest] | Manifest of key vehicle safety systems fitted to the vehicle |
| [Ignition cycle, crash] | The number (count) of power mode cycles at the time when the crash event occurred since the first use of the EDR. |
| [Ignition cycle, download] | The number (count) power mode cycles at the time when the data was downloaded since the first use of the EDR. |
| Vehicle Speed | The longitudinal speed of the vehicle that is calculated or estimated from the vehicle speed sensor (VSS). |
| Retarder Torque Mode | State signal which indicates which retarder torque mode is currently generating, limiting, or controlling retarder torque. |
| Brake Status – Parking | Indicates the status of the switch that is installed to detect whether or not the parking brake has been applied. |
| Brake Status – Service | Indicates the status of the switch that is installed in brake system to detect whether the service brake has been applied. This switch is usually used to turn on the brake lamps. |
| Engine Speed | Rotational speed of the engine output shaft. |
| Engine Load | Percent of available engine torque being generated |
| Accelerator Pedal Position | Ratio of the throttle pedal opening (driver’s operation) in percent. |
| ABS Brake Control Status – Tractor | Indicates the status of the ABS Brake control system on the vehicle/tractor, active or not active. |
| ABS Brake Control Status – Trailer | Indicates the status of the ABS Brake control system on Trailer(s), active or not active. Active if ABS Brake control is active for any trailer. |
| ACC Mode | Control status of Adaptive Cruise Control |
| Cruise Control States | The current state, or mode, of operation by the cruise control device |
| Automatic Emergency Braking | Forward Collision Advanced Emergency Braking System state |
| Time to Collision with Relevant Object [not required] | The time to collision is the duration after which the predicted travelling paths of host vehicle and relevant object lead to a distance of 0m between both. |
| Speed of Forward Vehicle [not required] | Absolute velocity of the preceding vehicle situated within 250 m in the same lane and moving in the same direction |
| Distance to Forward Vehicle [not required] | Distance to the preceding vehicle situated within 250 m in the same lane and moving in the same direction. |
| Lane Departure Warning System State | Indicates the status of Lane Departure Warning system. |
| Steering wheel angle | Angle of the steering shaft connected to driver control |
| Rollover Protection Engine Control | Stability control of engine retarder for rollover protection |
| Stability Control Braking [only on foundation brake application/control] | Stability control of wheel brakes for Roll Over Protection or Yaw Control |
| Yaw Control of Engine Retarder | Stability control of engine retarder for yaw control |
| Yaw Control of Wheel Brakes | Stability control of wheel brakes for yaw control |
| VDC System State | VDC Operational State (Can this be combined into a single signal with stability control above?) |
| Blind Spot Warning System Status | Operating status of the blind spot warning system |
| Crash [Impact Mitigation] System Activation Notification | Indicates detection and type of crash by installed crash mitigation system. |
| Seat Belt Status (Driver) | Shows if buckled or not |
| Seat Belt Status (Passenger) | Shows if buckled or not |
| Safety Restraint System Status | Operating status of safety restraint system(s) |
| [Delta-V, longitudinal] | The cumulative change in velocity, as recorded by the EDR of the vehicle, along the longitudinal axis. |
| [Maximum delta-V, longitudinal] | The maximum value of the cumulative change in velocity, as recorded by the EDR, of the vehicle along the longitudinal axis. |
| [Time, maximum delta-V, longitudinal] | The time from crash time zero to the point where the maximum value of the cumulative change in velocity is found, as recorded by the EDR, along the longitudinal axis. |
| [Maximum delta-V, lateral] | The maximum value of the cumulative change in velocity, as recorded by the EDR, of the vehicle along the lateral axis. |
| [Time, maximum delta-V, lateral] | The time from crash time zero to the point where the maximum value of the cumulative change in velocity is found, as recorded by the EDR, along the lateral axis. |
| [Maximum delta-V, resultant] | The maximum value of the cumulative change in velocity, as recorded by the EDR, of the vehicle along the lateral and longitudinal axis. |
| [Time, maximum delta-V, resultant] | The time from crash time zero to the point where the maximum value of the cumulative change in velocity is found, as recorded by the EDR, along the lateral and longitudinal axis. |
| [Roll angle] |  |
| [Roll rate] |  |
| [Pre-crash acceleration, longitudinal] | 10Hz |
| [Pre-crash acceleration, lateral] | 10Hz |
| Accident Emergency Call System Status | Operating status of emergency call system |
| VRU System Status | Operating status of the VRU system |
| Tyre Pressure Monitoring System Status | Operating status of the tire Pressure Monitoring System |

?  	Format requirements specified below are minimum requirements and manufacturers can exceed them.

?	 "Mandatory" is subject to the conditions detailed in Section 1.

? 	Pre-crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash time is -0.1 to 1.0 sec (e.g., T = -1 would need to occur between -1.1 and 0 seconds.)

? 	Accuracy requirement only applies within the range of the physical sensor. If measurements captured by a sensor exceed the design range of the sensor, the reported element shall indicate when the measurement first exceeded the design range of the sensor.